TSPL-EZ V5.23 Firmware Modification History

# TSPL2 Modifications

- Modify the problem of nested IF ELSEIF ENDIF loop.

- Use MONOTYPE TTF engine.

- Add codepage 1250, 1252, 1253, 1254, 1255, 1256, 1257, 1258, ISO-8859-1, ISO8859-2, ISO-8859-3, ISO-8859-4, ISO-8859-5, ISO-8859-6, ISO-8859-7, ISO-8859-8, ISO-8859-8, ISO-8859-9. ISO-8859-10, ISO-8859-15.

These codepages are only supported when using TTF font.

- Add centronic error status.

- Modify the problem of BITMAP command.

- Modify the checksum calculation of CODE 39C.

# EPL2 Emulation Modifications

- Modify the ES command.

- Modify the problem when print a label under continuous label mode, the last line will not be printed out.

- Modify the problem of p2 parameter of the command PA.

- Modify the problem of counter command C. Ex 999 will not advance to 1000.

- Modify the barcode 3C, 1A, 1B, and 1C.

- Modify the command WG.

- Clear the counter variable (C command) after a batch labels are printed.

- Modify the command xa to detect the sensor strength and measure the label length at the same time.

- Modify the WG command.

10. Modify the MAXICODE mode 3 input format.

11. Support Chinese bitmap font downloading and printing.

# ZPL2 Emulation Modifications

- Modify the problem when the first character in the expression of ^FD … ^FS and ^FV … ^FS is <CR> < LF> or <,>, the printer will stop receiving data.

- Support QRCODE.

- Modify the coordination problem when ^FT is going with ^FB command.

- Support the command ^EF and ^EG. The function of ^EF is as same as ~EF and the function of ^EG is as same as ~EG.